Hardware Integration Documentation

Module: Wi-Fi and Blynk IoT Remote Car Control using ESP32 and STM32

## 1.1 Objective

To implement remote control of a motor-driven car through Wi-Fi using the Blynk IoT platform. The ESP32 acts as the Wi-Fi/Blynk gateway and receives movement commands from the Blynk application. It forwards the commands to the STM32-based Custom Board through UART, while the STM32 performs the real-time motor-control operations using PWM and digital direction signals.

## 1.2 System Architecture

The application separates network/cloud communication from low-level vehicle control. The ESP32 handles Wi-Fi connectivity and Blynk IoT commands, while the STM32 remains responsible for motor direction, PWM speed control, command interpretation, safe startup, and emergency stopping.

Blynk IoT Mobile/Web Interface → Wi-Fi → ESP32

ESP32 → UART command → STM32

STM32 → PWM + direction signals → Motor Driver

Motor Driver → Left and Right DC Motors

## 1.3 Components Used

| Component | Specification |
| --- | --- |
| Primary Controller | STM32-based Custom Board running MicroPython |
| Wireless / IoT Controller | ESP32 running Arduino code |
| Cloud / Control Platform | Blynk IoT |
| Network Interface | ESP32 Wi-Fi |
| Motor Driver | Dual-channel motor driver with PWM enable and direction inputs |
| Left Motor | DC motor controlled through LEFT_IN1 and LEFT_IN2 |
| Right Motor | DC motor controlled through RIGHT_IN1 and RIGHT_IN2 |
| STM32 UART | UART1 at 115200 baud |
| STM32 PWM | ENA on A6, ENB on A7, 1000 Hz |

## 1.4 STM32 Pin Assignment Summary

| Function | STM32 GPIO | Mode | Purpose |
| --- | --- | --- | --- |
| UART1 TX | PA9 | UART | Transmit / communication |
| UART1 RX | PA10 | UART | Receive commands from ESP32 |
| Left Motor Enable (ENA) | A6 | PWM Output | Left motor speed |
| Right Motor Enable (ENB) | A7 | PWM Output | Right motor speed |
| Left Motor IN1 | B0 | GPIO Output | Left motor direction |
| Left Motor IN2 | B1 | GPIO Output | Left motor direction |
| Right Motor IN1 | C0 | GPIO Output | Right motor direction |
| Right Motor IN2 | C1 | GPIO Output | Right motor direction |

## 1.5 ESP32 Wi-Fi and Blynk Control Layer

The ESP32 provides the wireless command interface. The Blynk application supplies the user control inputs over Wi-Fi, and the ESP32 translates those control events into the command format expected by the STM32. The exact Blynk virtual-pin mapping and Wi-Fi credentials are defined by the supplied ESP32 Arduino source.

Blynk IoT provides the remote control interface.

ESP32 connects to the configured Wi-Fi network.

Blynk control events are processed by the ESP32.

Movement commands are forwarded to the STM32 through the serial/UART link.

The STM32 does not directly depend on the Blynk application for motor-control execution.

## 1.6 UART Communication Configuration

### 1.6.1 STM32 UART

UART1 is initialized at 115200 baud.

The source identifies PA9 as TX and PA10 as RX.

The STM32 continuously checks for received data using uart.any().

The STM32 reads one byte at a time and converts the command to uppercase.

### 1.6.2 ESP32-to-STM32 Command Path

Blynk generates a movement-control event on the ESP32.

The ESP32 converts the event into the corresponding vehicle command.

The command is sent over UART to the STM32.

The STM32 interprets the command as F, B, L, R, or S and executes the corresponding motor function.

## 1.7 Motor Control Configuration

### 1.7.1 PWM Speed Control

| Parameter | Value | Purpose |
| --- | --- | --- |
| PWM Frequency | 1000 Hz | Motor speed control |
| Straight Speed | 60000 / 65535 ≈ 91.6% | Forward motion |
| Turn Speed | 64000 / 65535 ≈ 97.7% | Turning |
| Backward Speed | 50000 / 65535 ≈ 76.3% | Reverse motion |

### 1.7.2 Motor Direction Control

| Command | Left Motor | Right Motor | Vehicle Behavior |
| --- | --- | --- | --- |
| F | IN1=1, IN2=0 | IN1=1, IN2=0 | Forward |
| B | IN1=0, IN2=1 | IN1=0, IN2=1 | Backward |
| L | IN1=0, IN2=1 | IN1=1, IN2=0 | Turn Left |
| R | IN1=1, IN2=0 | IN1=0, IN2=1 | Turn Right |
| S | IN1=0, IN2=0 | IN1=0, IN2=0 | Stop |

## 1.8 Functional Operation Description

1. The user operates the car controls from the Blynk IoT application.

2. The Blynk platform delivers the control event through the Wi-Fi connection to the ESP32.

3. The ESP32 processes the received control event and generates the appropriate single-character motor command.

4. The ESP32 transmits the command to the STM32 through the UART interface.

5. The STM32 continuously monitors UART data and reads the incoming command.

6. F causes both motors to rotate forward using STRAIGHT_SPEED.

7. B causes both motors to rotate backward using BACKWARD_SPEED.

8. L reverses the left motor and drives the right motor forward to turn the vehicle left.

9. R drives the left motor forward and reverses the right motor to turn the vehicle right.

10. S disables PWM and clears the motor direction outputs.

11. Unknown commands are reported by the STM32 without executing a movement function.

12. During startup, the STM32 executes stop_car() before accepting movement commands.

13. On KeyboardInterrupt, the STM32 executes stop_car() to disable the motors.

## 1.9 STM32 MicroPython Code

from machine import UART, Pin, PWM
import time


# ============================================================
# UART1 - ESP32 COMMUNICATION
# ============================================================

# STM32H563 MicroPython build uses the predefined UART1 pins.
#
# UART1:
# PA9  -> TX
# PA10 -> RX
#
# ESP32 TX must connect to STM32 PA10
# ESP32 RX must connect to STM32 PA9

uart = UART(1, 115200)

print()
print("========================================")
print("   STM32H563 CAR CONTROLLER")
print("========================================")
print("UART1 : 115200 baud")
print("TX    : PA9")
print("RX    : PA10")
print("========================================")


# ============================================================
# MOTOR DRIVER
# ============================================================

# ENA / ENB
# These are PWM pins.

ENA = PWM(Pin('A6'))
ENB = PWM(Pin('A7'))

# PWM frequency
ENA.freq(1000)
ENB.freq(1000)


# ============================================================
# MOTOR DIRECTION PINS
# ============================================================

# LEFT MOTOR
LEFT_IN1 = Pin('B0', Pin.OUT)
LEFT_IN2 = Pin('B1', Pin.OUT)

# RIGHT MOTOR
RIGHT_IN1 = Pin('C0', Pin.OUT)
RIGHT_IN2 = Pin('C1', Pin.OUT)


# ============================================================
# MOTOR SPEED
# ============================================================

# duty_u16 range:
#
# 0     = 0%
# 32768 = 50%
# 65535 = 100%
#
# ------------------------------------------------------------
# STRAIGHT SPEED
# ------------------------------------------------------------

STRAIGHT_SPEED = 60000
# Approximately 91.6%


# ------------------------------------------------------------
# TURNING SPEED
# ------------------------------------------------------------

TURN_SPEED = 64000
# Approximately 95.0%


# ------------------------------------------------------------
# BACKWARD SPEED
# ------------------------------------------------------------

BACKWARD_SPEED = 50000
# Approximately 76.3%


# ============================================================
# PWM SPEED FUNCTION
# ============================================================

def set_speed(speed):

    if speed < 0:
        speed = 0

    if speed > 65535:
        speed = 65535

    ENA.duty_u16(speed)
    ENB.duty_u16(speed)


# ============================================================
# STOP CAR
# ============================================================

def stop_car():

    # Stop PWM
    ENA.duty_u16(0)
    ENB.duty_u16(0)

    # Stop left motor
    LEFT_IN1.value(0)
    LEFT_IN2.value(0)

    # Stop right motor
    RIGHT_IN1.value(0)
    RIGHT_IN2.value(0)

    print("ACTION -> STOP")


# ============================================================
# FORWARD
# ============================================================

def move_forward():

    # Left motor forward
    LEFT_IN1.value(1)
    LEFT_IN2.value(0)

    # Right motor forward
    RIGHT_IN1.value(1)
    RIGHT_IN2.value(0)

    # STRAIGHT SPEED
    set_speed(STRAIGHT_SPEED)

    print(
        "ACTION -> FORWARD | SPEED =",
        STRAIGHT_SPEED
    )


# ============================================================
# BACKWARD
# ============================================================

def move_backward():

    # Left motor backward
    LEFT_IN1.value(0)
    LEFT_IN2.value(1)

    # Right motor backward
    RIGHT_IN1.value(0)
    RIGHT_IN2.value(1)

    # BACKWARD SPEED
    set_speed(BACKWARD_SPEED)

    print(
        "ACTION -> BACKWARD | SPEED =",
        BACKWARD_SPEED
    )


# ============================================================
# TURN LEFT
# ============================================================

def turn_left():

    # Left motor backward
    LEFT_IN1.value(0)
    LEFT_IN2.value(1)

    # Right motor forward
    RIGHT_IN1.value(1)
    RIGHT_IN2.value(0)

    # TURN SPEED
    set_speed(TURN_SPEED)

    print(
        "ACTION -> LEFT | SPEED =",
        TURN_SPEED
    )


# ============================================================
# TURN RIGHT
# ============================================================

def turn_right():

    # Left motor forward
    LEFT_IN1.value(1)
    LEFT_IN2.value(0)

    # Right motor backward
    RIGHT_IN1.value(0)
    RIGHT_IN2.value(1)

    # TURN SPEED
    set_speed(TURN_SPEED)

    print(
        "ACTION -> RIGHT | SPEED =",
        TURN_SPEED
    )


# ============================================================
# SAFE STARTUP
# ============================================================

stop_car()

print("CAR READY")
print("Waiting for commands...")
print()
print("F = Forward")
print("B = Backward")
print("L = Left")
print("R = Right")
print("S = Stop")
print()
print("STRAIGHT SPEED :", STRAIGHT_SPEED)
print("TURN SPEED     :", TURN_SPEED)
print("BACKWARD SPEED :", BACKWARD_SPEED)
print()


# ============================================================
# MAIN CONTROL LOOP
# ============================================================

try:

    while True:

        # Check if ESP32 sent something
        if uart.any():

            data = uart.read(1)

            if data:

                try:
                    command = data.decode('utf-8').upper()

                except:
                    command = ''

                if command:

                    print(
                        "UART COMMAND ->",
                        repr(command)
                    )


                    # ----------------------------------------
                    # FORWARD
                    # ----------------------------------------

                    if command == 'F':

                        move_forward()


                    # ----------------------------------------
                    # BACKWARD
                    # ----------------------------------------

                    elif command == 'B':

                        move_backward()


                    # ----------------------------------------
                    # LEFT
                    # ----------------------------------------

                    elif command == 'L':

                        turn_left()


                    # ----------------------------------------
                    # RIGHT
                    # ----------------------------------------

                    elif command == 'R':

                        turn_right()


                    # ----------------------------------------
                    # STOP
                    # ----------------------------------------

                    elif command == 'S':

                        stop_car()


                    # ----------------------------------------
                    # UNKNOWN COMMAND
                    # ----------------------------------------

                    else:

                        print(
                            "WARNING -> Unknown command:",
                            repr(command)
                        )


        time.sleep_ms(10)


# ============================================================
# EMERGENCY STOP
# ============================================================

except KeyboardInterrupt:

    stop_car()

    print()
    print("========================================")
    print("CAR CONTROLLER STOPPED")
    print("MOTORS DISABLED")
    print("========================================")

## 1.10 ESP32 Arduino / Blynk Code

#define BLYNK_TEMPLATE_ID "TMPL3OqXnyg2Z"
#define BLYNK_TEMPLATE_NAME "Quickstart Template"
#define BLYNK_AUTH_TOKEN "K8S19qaLdPkd0hnwFYEclm5GmNB_zJkV"

#include <WiFi.h>
#include <BlynkSimpleEsp32.h>

char ssid[] = "Chatur_2";
char pass[] = "ABCD1234";

#define RXD2 16
#define TXD2 17

HardwareSerial STM(2);

int joyX = 512;
int joyY = 512;

char lastCommand = 'S';

const int CENTER = 512;
const int DEADZONE = 150;

void sendCommand(char cmd)
{
    if(cmd != lastCommand)
    {
        STM.write(cmd);
        lastCommand = cmd;

        Serial.print("Sent : ");
        Serial.println(cmd);
    }
}

void processJoystick()
{
    Serial.print("X=");
    Serial.print(joyX);
    Serial.print("  Y=");
    Serial.println(joyY);

    char command = 'S';

    if(joyY > CENTER + DEADZONE)
    {
        command = 'F';
    }
    else if(joyY < CENTER - DEADZONE)
    {
        command = 'B';
    }
    else if(joyX < CENTER - DEADZONE)
    {
        command = 'L';
    }
    else if(joyX > CENTER + DEADZONE)
    {
        command = 'R';
    }
    else
    {
        command = 'S';
    }

    sendCommand(command);
}

BLYNK_WRITE(V0)
{
    joyX = param.asInt();
    processJoystick();
}

BLYNK_WRITE(V1)
{
    joyY = param.asInt();
    processJoystick();
}

void setup()
{
    Serial.begin(115200);

    STM.begin(115200, SERIAL_8N1, RXD2, TXD2);

    Serial.println();
    Serial.println("==============================");
    Serial.println("SOURYA REMOTE CAR");
    Serial.println("==============================");

    Serial.print("Connecting WiFi");

    WiFi.begin(ssid, pass);

    while(WiFi.status() != WL_CONNECTED)
    {
        delay(500);
        Serial.print(".");
    }

    Serial.println();
    Serial.println("WiFi Connected");
    Serial.print("IP : ");
    Serial.println(WiFi.localIP());

    Blynk.config(BLYNK_AUTH_TOKEN);

    Serial.print("Connecting Blynk");

    while(!Blynk.connect())
    {
        Serial.print(".");
        delay(500);
    }

    Serial.println();
    Serial.println("Blynk Connected");
    Serial.println("Ready");
}

void loop()
{
    Blynk.run();

    static unsigned long timer = 0;

    if(millis() - timer > 5000)
    {
        timer = millis();

        Serial.println("----------------------");

        if(WiFi.status() == WL_CONNECTED)
        {
            Serial.println("WiFi : Connected");
            Serial.print("RSSI : ");
            Serial.println(WiFi.RSSI());
        }
        else
        {
            Serial.println("WiFi : Disconnected");
        }

        if(Blynk.connected())
        {
            Serial.println("Blynk : Connected");
        }
        else
        {
            Serial.println("Blynk : Disconnected");
        }
    }
}

## 1.11 Command Mapping

| Blynk Control | UART Command | STM32 Function | Vehicle Action |
| --- | --- | --- | --- |
| Forward control | F | move_forward() | Forward |
| Backward control | B | move_backward() | Backward |
| Left control | L | turn_left() | Left turn |
| Right control | R | turn_right() | Right turn |
| Stop control | S | stop_car() | Stop |

## 1.12 Serial Output / Verification

STM32 startup: STM32H563 CAR CONTROLLER

STM32: UART1 : 115200 baud

STM32: CAR READY

STM32: Waiting for commands...

STM32: UART COMMAND -> '<command>'

STM32: ACTION -> FORWARD | SPEED = 60000

STM32: ACTION -> BACKWARD | SPEED = 50000

STM32: ACTION -> LEFT | SPEED = 64000

STM32: ACTION -> RIGHT | SPEED = 64000

STM32: ACTION -> STOP

STM32 emergency shutdown: CAR CONTROLLER STOPPED / MOTORS DISABLED

## 1.13 Known Limitation

The supplied STM32 implementation uses fixed speed values for forward, turning, and backward movement.

The STM32 control logic supports only the defined single-character movement commands.

UART command reception on the STM32 is polling-based rather than interrupt- or DMA-based.

The supplied source does not define a formal command acknowledgement, checksum, sequence number, or communication timeout.

The exact Blynk virtual-pin assignments, dashboard layout, and network configuration are determined by the supplied ESP32 source.

The supplied source does not specify the motor-driver model, motor electrical ratings, or vehicle battery specifications.

## 1.14 Safety and Reliability Considerations

The STM32 starts with stop_car(), ensuring the motors are disabled before command processing begins.

The S command provides an explicit software stop function.

KeyboardInterrupt causes stop_car() to be executed as an emergency software shutdown.

Ensure the ESP32-to-STM32 UART signal levels are electrically compatible.

Do not power the motors directly from STM32 GPIO pins; use an appropriate motor driver and power supply.

Provide a common signal ground between the ESP32, STM32, and motor-driver control interface as required.

For a practical remote vehicle, communication-loss handling should be implemented so the vehicle does not continue moving if the network or ESP32 link fails.

## 1.15 Readiness for Future Enhancement

Add a communication watchdog on the STM32 that automatically stops the car when commands are not received within a defined timeout.

Add Blynk telemetry for battery voltage, motor status, distance, and sensor information.

Add variable-speed control through Blynk instead of fixed speed presets.

Add obstacle detection using the Custom Board's ultrasonic and IR sensors.

Add autonomous/manual hybrid driving modes.

Use UART interrupts or DMA for deterministic high-reliability communication.

Add command acknowledgements and fault reporting from STM32 to ESP32 and then to Blynk.

End of Document